## Network Documentation

### Network Overview

The network comprises multiple VLANs linked through a core switch, with on-premise and cloud-based servers. It is designed to support various departments and functionalities, including operational control, laboratories, IT services, treatment ponds, and IoT devices.

### VLAN Configuration

- VLAN 10 - OCR (Operational Control Room): 50 computers

  - Subnet: 192.168.10.0/24

- VLAN 20 - Lab (Laboratory): 25 computers

  - Subnet: 192.168.20.0/24

- VLAN 30 - IT (Information Technology): 25 computers

  - Subnet: 192.168.30.0/24

- VLAN 40 - Treatment Pond: 200 sensors

  - Subnet: 192.168.40.0/24

- VLAN 50 - Servers: Various on-premise servers

  - Subnet: 192.168.50.0/24

- VLAN 60 - IoT: IoT devices and servers

  - Subnet: 192.168.60.0/24

### Servers and IP Configuration

- On-Premise Servers:

  - Authorization Server: 192.168.1.4

  - Email Server: 192.168.1.6

  - DNS Server: 192.168.3

  - File Server: 192.168.1.7

  - SCADA Sensor Data Server: 192.168.1.2

- Cloud Servers:

  - Web Server: 100.100.100.1

  - IoT Server: 200.200.200.1

- DHCP Server: 192.168.1.2 (Serving all VLANs)

### Network Devices

- Core Switch: Connects all VLANs and servers and Manages inter-VLAN routing.

- Router: Connection to the internet.

### Configuration Steps

#### 1. Core Switch Configuration

- VLAN Setup:

plaintext

Copy code

Switch> enable

Switch# configure terminal

! Creating VLANs

Switch(config)# vlan 10

Switch(config-vlan)# name OCR

Switch(config-vlan)# exit

Switch(config)# vlan 20

Switch(config-vlan)# name Lab

Switch(config-vlan)# exit

Switch(config)# vlan 30

Switch(config-vlan)# name IT

Switch(config-vlan)# exit

Switch(config)# vlan 40

Switch(config-vlan)# name Treatment-Pond

Switch(config-vlan)# exit

Switch(config)# vlan 50

Switch(config-vlan)# name Servers

Switch(config-vlan)# exit

Switch(config)# vlan 60

Switch(config-vlan)# name IoT

Switch(config-vlan)# exit

! Assigning ports to VLANs

Switch(config)# interface range fa0/1 - 50

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

Switch(config-if-range)# exit

Switch(config)# interface range fa0/51 - 75

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

Switch(config-if-range)# exit

Switch(config)# interface range fa0/76 - 100

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 30

Switch(config-if-range)# exit

Switch(config)# interface range fa0/101 - 200

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 40

Switch(config-if-range)# exit

Switch(config)# interface range fa0/201 - 250

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 50

Switch(config-if-range)# exit

Switch(config)# interface range fa0/251 - 300

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 60

Switch(config-if-range)# exit

#### 2. OSPF Configuration

- OSPF Routing:

plaintext

Copy code

Router(config)# router ospf 1

Router(config-router)# network 192.168.10.0 0.0.0.255 area 0

Router(config-router)# network 192.168.20.0 0.0.0.255 area 0

Router(config-router)# network 192.168.30.0 0.0.0.255 area 0

Router(config-router)# network 192.168.40.0 0.0.0.255 area 0

Router(config-router)# network 192.168.50.0 0.0.0.255 area 0

Router(config-router)# network 192.168.60.0 0.0.0.255 area 0

Router(config-router)# exit

#### 3. Static Routes for Cloud Servers

- Static Routing:

plaintext

Copy code

Router(config)# ip route 100.100.100.0 255.255.255.0 192.168.1.1

Router(config)# ip route 200.200.200.0 255.255.255.0 192.168.1.1

### Network Testing and Validation

- Ping Test:

  - Verify connectivity between different VLANs by pinging from PCs in one VLAN to another.

  - Test connectivity to on-premise servers and cloud servers.

- Role-Based Access Testing:

  - Ensure role-based access controls are functioning as expected, allowing only authorized access to specific VLANs.

- Network Services Validation:

  - Test DHCP server by checking if PCs are receiving the correct IP addresses.

  - Verify DNS resolution and email services.

- Hint:

All vlans must be configured in all Switches.

### Conclusion

This documentation provides a comprehensive guide to setting up a network with multiple VLANs, on-premise and cloud-based servers, using static routing and OSPF for internet traffic. The detailed configuration steps ensure a robust and efficient network design tailored to the specified requirements. If you have any questions or need further assistance, feel free to ask!